Coefficient of Variation (CV) Report

1. Purpose

In RFID security, frequency and magnitude can often be spoofed by high-end emulators. Coherence, measured via the Coefficient of Variation (CV), identifies the "texture" of the signal. This report details how this value is mathematically obtained to distinguish between the stable resonance of a passive card and the synthetic jitter of an electronic intruder.

2. Mathematical Acquisition Process

The acquisition of the Complexity metric follows a four-step digital signal processing pipeline:

Step 1: Spectral Transformation (FFT) The raw time-domain signal is converted into the frequency domain using a Fast Fourier Transform (FFT). This allows us to see the distribution of power across different frequencies.

Step 2: Peak Extraction (n=10) The system identifies the Top 10 most prominent magnitudes (m) in the spectrum. Focusing on a set of peaks rather than a single carrier frequency allows the system to measure the "stability" of the signal's harmonics.

Set = {m1, m2, m3, …., m10}

Step 3: Statistical Moments Calculation The script calculates the arithmetic Mean (µ) and the Standard Deviation (σ) of these 10 peaks.

- Mean (µ): Represents the average strength of the signal's primary components.

- Standard Deviation (σ): Represents the "spread" or "erraticism" of those components.

Step 4: Coefficient of Variation (CV) Calculation Coherence is defined as the ratio of the deviation to the mean. This "normalizes" the data, ensuring that a high-power signal and a low-power signal can be compared on the same scale of "cleanliness."

CV = µ / σ

3. Data Analysis & Threshold Establishment

By running the comprehensive_analysis.py script across all test groups, we observed a distinct mathematical gap between authorized hardware and anomalies.

| Device Category | Internal Logic | Observed CV Range | Signal Profile |
| --- | --- | --- | --- |
| Standard Card | Passive Induction | 1.65 – 1.79 | High Coherence (Uniform Peaks) |
| Emulator (Anomaly) | Synthetic Synthesis | 1.84 – 1.98 | Low Coherence (Erratic Peaks) |

Deriving the Threshold (Tcv): To establish the digital footprint boundary, we calculated the midpoint between the "noisiest" legitimate card and the "cleanest" anomaly:

Tcv): =1.72Max Card + 1.84Min Anomaly2 = 1.815

We established the final IDS threshold at 1.82.

4. Visualization of Coherence

- Low Complexity (Authorized): The peaks are uniform in height. Because a passive card coil has a fixed physical resonance, the standard deviation (σ) is low, resulting in a CV below 1.82.

- High Complexity (Intrusion): The peaks are erratic. Hardware emulators like the Proxmark3 introduce "spectral splatter" and jitter. This creates a high standard deviation (σ), resulting in a CV above 1.82.

5. Conclusion

The Coherence (CV) metric is the most effective tool in our IDS for detecting sophisticated hardware emulators. While an attacker can adjust their software to match our Location (Frequency) and Intensity (Magnitude), they cannot easily replicate the exact "Physical Layer Coherence" of a passive silicon chip. By mathematically enforcing a CV limit of 1.82, we successfully filter out synthetic signals that lack the stable resonance of an authorized card.