Digital Footprint

The digital footprint is essentially a three-dimensional boundary box. If a signal falls inside the box, it is "Authorized"; if it falls outside, it is an "Anomaly." Here is the mathematical breakdown of how specific results defined each wall of that box.

1. Setting the "Location" (Frequency)

The location is set using a Range Analysis. We look at all the frequencies the authorized cards produced across the groups and establish a minimum and maximum threshold.

Mathematical Observation:

- Authorized Values (A): cards showed medians at 875 kHz, 750 kHz, and 2243.75 kHz.

- Anomaly Values (X): The Flipper Zero and Proxmark3 showed values like 381.25 kHz (too low) and 3487.50 kHz (too high).

Derivation:

We set the "Lower Wall" just below the lowest card and the "Upper Wall" just above the highest card.

- Lower Bound: min(A) – buffer = 750 - 10 = 740 kHz

- Upper Bound: (A) + buffer = 2243.75 + 56.25 = 2300 kHz

2. Setting the "Intensity" (Magnitude)

Intensity is the Power Threshold. Because a standard card is passive (it only reflects energy), it has a physical "ceiling" for how much power it can show on the oscilloscope.

Mathematical Observation:

- Cards never exceeded a peak magnitude of 0.2252 (seen in Group 3-C).

- Some anomalies, like the Flipper, reached 0.2242, while others were lower.

Derivation:

To ensure a legitimate card never accidentally triggers a fail (False Positive), we set the threshold (Tm)  roughly 10% higher than the maximum legitimate card seen in testing.

Tm = 0.2252 x 1.1 = 0.25

- Logic: If a signal is stronger than 0.25, it is physically impossible for it to be a passive card at the current sensor distance; it must be an active, powered device.

3. Setting the "Coherence" (Complexity / CV)

This is the most critical part of the footprint because it detects the Hardware Signature. We use the Coefficient of Variation (CV) to measure how much the signal's top peaks vary.

Mathematical Observation:

- Authorized Card CVs: {1.6695, 1.7828, 1.7342} — All are below 1.80.

- Anomaly CVs: 1.8662, 1.8473, 1.9816} — All are above 1.84.

Derivation:

There is a clear mathematical "gap" between 1.78 (highest card) and 1.84 (lowest anomaly). We set the threshold (Tcv) in the middle of that gap to create a safety margin for both sides.

Tcv =1.78 +1.84 2 = 1.81

(I used 1.82 in my script, which provides a slightly larger buffer for the cards).

How the Data Establishes the Footprint (Summary)

By combining these three specific numbers, you created a Boolean Logic Filter. For a file to be "Authentic," it must satisfy this mathematical proof:

Verdict = (740 <= Freq <= 2300) AND (Mag <= 0.25) AND (CV <= 1.82)

This approach is powerful because even if an attacker manages to spoof the correct Frequency, they will likely fail the Complexity check due to the hardware noise of their emulator.