Final Report

1. Executive Summary

The objective of this analysis was to develop a robust, non-cryptographic method for identifying unauthorized RFID emulation devices (e.g., Flipper Zero, Proxmark3). By analyzing the physical layer of the radio frequency signal using Fast Fourier Transforms (FFT), we established a Digital Footprint consisting of three pillars: Location (Frequency), Intensity (Magnitude), and Coherence (Complexity). The resulting IDS (Intrusion Detection System) achieved 100% accuracy in distinguishing authentic cards from hardware emulators across three distinct test groups.

2. Methodology: Defining the "Ground Truth"

To build a detection system, we first characterized the "Authorized" signal using the fft_median_test.py script on baseline files.

The Baseline Fingerprint

- Median Frequency: 1243.75 kHz

- Median Magnitude: 0.0064

- Median Complexity (CV): 1.7099

This baseline represents the "DNA" of a legitimate passive RFID card. Any significant deviation from these values indicates a potential hardware-based attack.

3. The Digital Footprint Table

The following thresholds were mathematically derived to create the "Decision Boundary" for the IDS.

| Metric | Scientific Property | Authorized Boundary | Anomaly Logic |
| --- | --- | --- | --- |
| Location | Frequency Center | 740.0 – 2300.0 kHz | Detects Software/Clock Drift |
| Metric | Peak Magnitude | <= 0.2500 | Detects Active Power Emulators |
| Intensity | Complexity (CV) | <= 1.8200 | Detects Hardware Jitter/Noise |

4. Deep Dive: The Mathematical Pillars

I. Location (Frequency Range)

RFID readers and cards operate on specific resonance. However, software-defined tools like the Flipper Zero often exhibit "Frequency Shift" due to internal clock inaccuracies.

- Mathematical Proof: We set a window [740, 2300] kHz.

- Observation: In Group 3-B, the emulator drifted to 3487.50 kHz, immediately triggering a "Frequency Shift" alert.

II. Intensity (Magnitude)

Passive cards are unpowered; they merely reflect a portion of the reader's signal. Active emulators use batteries to broadcast a stronger signal to increase range.

- Mathematical Proof: The highest recorded legitimate card power was 0.2252. We established a ceiling of 0.2500.

- Observation: Any signal exceeding 0.25 is physically impossible for a passive card at the test distance, identifying it as a powered cloner.

III. Coherence (Complexity / CV)

This is the most critical metric for detecting high-end tools like the Proxmark3. We use the Coefficient of Variation (CV), calculated as:

CV = µ / σ

(Where σ is the standard deviation and µ is the mean of the top 10 signal peaks).

- Logic: Authentic cards produce very stable, uniform peaks (Low CV). Emulators produce "noisy" or "jittery" peaks (High CV).

- Observation: Authorized cards stayed below 1.79, while Proxmark3 samples consistently jumped above 1.85, allowing for a clear decision boundary at 1.82.

5. Experimental Results

Using the final_v2_test.py script, the system was validated against unknown samples:

- Authentic Cards (Pass): Corrected identified in Group 1-A, 2-B, and 3-C.

- Flipper Zero (Fail): Detected primarily via Frequency Shift.

- Proxmark3 (Fail): Detected primarily via Spectral Complexity (Noise).

6. Conclusion

By shifting security focus from the data layer to the Physical Layer, we successfully created a system capable of fingerprinting hardware. Even if an attacker perfectly clones the data on an RFID card, they cannot easily replicate the physical "Coherence" and "Intensity" signature of the original passive silicon, making this IDS a highly effective secondary layer of defense for secure facilities.